## Announcement Differential pressure transmitter P 26.2

Configurable pressure transmitter with automatic zero-point correction

Product description

- High-precision differential pressure transducer for cleanrooms, air

conditioning and process technology

- Precise measuring ranges from 10 Pa uni- and bidirectional

- No zero point drift thanks to automatic zero point adjustment

- Simple commissioning and variable installation (top-hat rail / wall mounting)

- Several switching and analog outputs as standard

- Extensive configuration options using free parameterization software via internal USB-C interface

Optional

- Available with a factory calibration certificate or a calibration certificate in accordance with DKD Guideline 6-1

- M12 connectors instead of cable glands

- Selectable power supply: 24 V AC/DC with or without galvanic isolation,
or mains voltage 100 V – 240 / 277 V

- Various pressure connections

- Display with buttons for local configuration

- Two optional relays enable the connection of mains-powered alarms when limit values are exceeded or not met

- T- or P-/T-compensation for precise volume flow measurement, 
including air consumption meters

Technical data

Measurement ranges:	10/ 50/ 100/ 250/ 500 Pa, 1 / 2.5 / 5/ 10 / 20 / 50 / 100 kPa

(also ± measurement ranges) freely scalable from 10 .. 100%

within a measurement range (others available upon request)

Measurement accuracy:	± 0.2 % FS measurement range ≤ 50 kPa or ± 0.5 % FS

Temperature coefficient span:	max. 0.03 % / K

Temperature coefficient zero point:	±0 % / K (cyclical/manual zero-point correction)

Max. system pressure	400 kPa for measurement ranges ≥ 2.5 kPa

Overload capacity:		200 x for measurement ranges < 2.5 kPa

Medium:		air, all non-aggressive and non-flammable gases

Step response time (T63):	25 ms .. 60 s (adjustable)

(Time constant)
Rated temperature range:	10 .. 50 ° C

Storage temperature:	- 10 .. 70 ° C

Power consumption:	DC: max. 4 W | AC: max. 8 VA

Weight:		approx. 750 g

Cable glands:		3 x M 16 clamping range Ø 5 ... 10 mm

Pressure ports:		see order code others on request

IP rating:		IP 65 according to IEC / EN 60529

Protection class:		II according to IEC / EN 61140

Overvoltage category:	II according to IEC / EN 60664-1

Pollution degree:		2 according to IEC / EN 60664-1

Certificates:		CE, NRTL according to UL/CSA 61010-1 in preparation

Designation:	Differential pressure transmitter

Type: 	P 26.2

Manufacturer: 	halstrup-walcher GmbH