## Announcement Differential pressure transmitter P26

Configurable pressure transmitter with automatic zero-point correction

- Measurement ranges from 0 .. 10 Pa to 100 kPa

- For measuring positive or negative pressures, even asymmetrically

- Scalable measurement ranges and units

- Measurement ranges / outputs can be configured

- Automatic zero-point calibration prevents zero-point drift

- Built-in valve provides a high level of overload protection

- Inductive measurement cell for highly accurate measurement and temperature stability

- Can be calibrated (ex works and on-site)

- Also for volume / mass flow and flow rate measurements with immediate display of the values

- Multilingual operating menu

- With 2 alarm contacts

- With multicolour LCD

- USB port

- Top hat rail assembly and wall mounting, IP 65

Technical data

Measurement ranges:	10/ 50/ 100/ 250/ 500 Pa, 1 / 2.5 / 5/ 10 / 20 / 50 / 100 kPa (others available upon request) freely scalable from 10 .. 100% within a measurement range

Outputs: 		0 .. 10 V, 0 .. 20 mA, 4 .. 20 mA, ± 5 V

Power supply:		24 VAC/DC, 24 VAC, 230/115 VAC

LCD: 		Multicolour + keyboard

Contact points: 		2 relays max. 230 VAC, 6 A

Measurement accuracy: 	±0.2%FS (for measurement ranges ≤ 50 kPa) or ±0.5% FS

Temperature coefficient span:	max. 0.03% FS/K (10 .. 50°C)

Temperature coefficient zero point:	± 0 % (cyclical/manual zero-point correction)

Max. system pressure

Overload capacity: 	600 kPa for measurement ranges ≥2.5 kPa
200x for measurement ranges <2.5 kPa

Medium: 		Air, all non-aggressive and non-flammable gases

Sensor response time: 	25 ms

Step response time (T63)

(Time constant):		25 ms.. 60 s (adjustable)

Rated temperature range:	10°C .. 50°C,

Storage temperature: 	-10°C .. 70 °C

Power consumption: 	approx. 6 VA

Weight: 		approx. . 750 g

Cable glands: 		3 x M 16

Pressure ports: 	see order code, others on request,
clamping range Ø 5 ... 10 mm

Protection class: 		IP 65, with USB IP 40

Housing/cover: 		ABS

Dimensions: 		164 mm x 102.5 mm x 76 mm

Wall mounted housing: 	Top hat rail assembly and wall mounting possible

Certificates: 		CE / UKCA, NRTL according to UL/CSA 61010-1

Data interface: 		USB, data cable supplied, External zero-point calibration

Optional: 		DAkkS calibration certificate, ISO factory calibration certificate

Designation:	Differential pressure transmitter

Type: 	P 26

Manufacturer: 	halstrup-walcher GmbH