## Announcement Differential pressure transmitter P 29.2

Configurable pressure transmitter with automatic zero-point correction

Product description

- High-precision differential pressure transmitter for natural gas

- Precisely matched measurement ranges starting at 250 Pa

- No zero-point drift thanks to automatic zero-point calibration

- Easy commissioning and flexible mounting (DIN rail and wall mounting)

- Multiple digital and analog outputs as standard

- Extensive configuration options via parameterization software using the internal USB-C interface

Optional

- Available with a factory calibration certificate or a calibration certificate in accordance with DKD Guideline 6-1

- M12 connector instead of cable glands

- 24 V AC/DC with or without galvanic isolation

- Clamp ring fitting instead of a laboratory connector

- Display with buttons for configuring the device

Technical data

Measurement ranges:	250 / 500 Pa, 1 / 2.5 / 5 / 10 kPa

(others available upon request) freely scalable from

10 .. 100 % within a measurement range

Measurement accuracy:	± 0.2 % FS or ± 0.5 % FS

Temperature coefficient span: 	max. 0.03 % / K

Temperature coefficient zero point:	±0% / K (cyclical/manual zero-point correction)

Max. system pressure:	at least 200 times, but a maximum of 100 kPa

Medium:		air, all non-aggressive gases and natural gas

Max. system pressure:	100 kPa for all measurement ranges

Step response time (T63):	25 ms .. 60 s (adjustable)

(Time constant)

Rated temperature range.	10 .. 50 ° C

Storage temperature:	- 10 .. 70 ° C

Power consumption:	DC: max. 4 W | AC: max. 8 VA

Weight:		approx. 750 g

Cable glands:		2 x M 16 clamping range Ø 5 ... 10 mm

IP rating:		IP 65 according to IEC / EN 60529

Protection class:		II according to IEC / EN 61140

Overvoltage category:	II according to IEC / EN 60664-1

Pollution degree:		2 according to IEC / EN 60664-1

Certificates:	CE, EN1127-1:2019,
NRTL according to UL/CSA 610101 in preparation

Designation:	Differential pressure transmitter

Type: 	P 29.2

Manufacturer: 	halstrup-walcher GmbH